INT 09h (9)              Keyboard

The keyboard generates an INT 9 every time a key is pushed or

released.

Notes:         This is a hardware interrupt (IRQ 1) activated by

the make or break of every keystroke.

The default INT 9 handler in the ROM reads the make and break scan

codes from the keyboard and converts them into actions or key codes as

follows:

ş   For ASCII keys, when a make code is encountered, the ASCII code

and the scan code for the key are placed in the 32-byte keyboard

buffer, which is located at 0:41Eh. The ASCII code and scan code

are placed in the buffer at the location addressed by the

Keyboard Buffer Tail Pointer (0:041Ch). The Keyboard Buffer Tail

Pointer is then incremented by 2, and if it points past the end

of the buffer, it is adjusted so that it points to the beginning

of the buffer.

ş   If Ctrl, Alt, or Shift has been pressed, the Shift Status

(0:0417h) and Extended Shift Status (0:0418h) bytes are updated.

ş   If the Ctrl-Alt-Del combination has been pressed, the Reset

Flag (0:0472h) is set to 1234h and control is given to the

power-on self test (POST). Because the Reset Flag is 1234h, the

POST routine bypasses the memory test.

ş   If the Pause key sequence has been entered, this interrupt

enters an indefinite loop. The loop is broken as soon as a valid

ASCII keystroke is entered. (The PC Convertible issues an INT

15h, Service 41h (Wait on External Event), to execute its pause

loop.)

ş   If the Print Screen key sequence is entered, an INT 05h (Print

Screen) is executed.

ş   If the Control-Break key sequence is entered, an INT 1Bh

(Control-Break) is executed.

ş   For XTs dated 1/10/86 and after, ATs, XT-286s, and PC

Convertibles, the INT 9h handler generates an INT 15h, function

91h (Interrupt Complete) to signal that a keystroke is

available. Also, on these machines, a make or break of the Sys

Req key generates an INT 15h, function 85h (System Request Key

Pressed).

ş   For ATs dated 6/10/85 and after, XT-286s, and PC Convertibles,

an INT 15h, function 4Fh (Keyboard Intercept) is executed after

the scan code has been read from the keyboard port (60h). This

allows the user to redefine or remove a keystroke.

INT 16 provides a standard way to read characters from the keyboard

buffer that have been placed there by the INT 9 handler in ROM.

INT 16h,  11h (17)       Extended Keyboard Status                        many

Checks to see if a character is available in the buffer. This service

is provided to support the extended keyboard (101/102-key keyboard).

On entry:      AH         11h

Returns:       Zero       0, if character is available

1, if character is not available

AX         If character is available, then set to

character as in Service 10h

--------------------------------------------------------------------------

Notes:         This service is supported only on the AT dated

11/15/85 and after, the XT dated 1/10/86 and after,

and the XT-286.

If a character is available, the Zero Flag is

cleared and AX contains the ASCII value in AL and

the scan code in AH. The character is not removed

from the buffer. Use Service 10h to remove the

character from the buffer. See Service 10h for a

complete description of the meaning of AX if a

character is available.

This service is excellent for clearing the keyboard

or allowing a program to be interruptable by a

specific key sequence.

See Service 01h for an equivalent service that works

with all keyboards, not supporting the new keys on

the enhanced (101/102-key) keyboard.

Keyboard Codes

This tables lists all the keyboard codes that are returned by calls to

INT 16 functions 0 through 2 (the second and third columns) and 10h

through 12h (the last column).

83/84-Key[1]  101/102-Key[2]  101/102-Key

Standard       Standard       Extended

Keystroke                 Function     Function[3]    Function[4]

Esc                        01/1B           same           same

1                          02/31           same           same

2                          03/32           same           same

3                          04/33           same           same

4                          05/34           same           same

5                          06/35           same           same

6                          07/36           same           same

7                          08/37           same           same

8                          09/38           same           same

9                          0A/39           same           same

0                          0B/30           same           same

-                          0C/2D           same           same

=                          0D/3D           same           same

Backspace                  0E/08           same           same

Tab                        0F/09           same           same

q                          10/71           same           same

w                          11/77           same           same

e                          12/65           same           same

r                          13/72           same           same

t                          14/74           same           same

y                          15/79           same           same

u                          16/75           same           same

i                          17/69           same           same

o                          18/6F           same           same

p                          19/70           same           same

[                          1A/5B           same           same

]                          1B/5D           same           same

Enter                      1C/0D           same           same

Ctrl                         **             **             **

a                          1E/61           same           same

s                          1F/73           same           same

d                          20/64           same           same

f                          21/66           same           same

g                          22/67           same           same

h                          23/68           same           same

j                          24/6A           same           same

k                          25/6B           same           same

l                          26/6C           same           same

;                          27/3B           same           same

'                          28/27           same           same

`                          29/60           same           same

Shift                        **             **             **

\                          2B/5C           same           same

z                          2C/7A           same           same

x                          2D/78           same           same

c                          2E/63           same           same

v                          2F/76           same           same

b                          30/62           same           same

n                          31/6E           same           same

m                          32/6D           same           same

,                          33/2C           same           same

.                          34/2E           same           same

/                          35/2F           same           same

Gray *                     37/2A           same           same

Alt                          **             **             **

Space                      39/20           same           same

Caps Lock                    **             **             **

F1                         3B/00           same           same

F2                         3C/00           same           same

F3                         3D/00           same           same

F4                         3E/00           same           same

F5                         3F/00           same           same

F6                         40/00           same           same

F7                         41/00           same           same

F8                         42/00           same           same

F9                         43/00           same           same

F10                        44/00           same           same

F11                        no key           --           85/00

F12                        no key           --           86/00

Num Lock                     **             **             **

Scroll Lock                  **             **             **

White Home                 47/00           same           same

White Up Arrow             48/00           same           same

White PgUp                 49/00           same           same

Gray -                     4A/2D           same           same

White Left Arrow           4B/00           same           same

Center Key                   --             --           4C/00

White Right Arrow          4D/00           same           same

Gray +                     4E/2B           same           same

White End                  4F/00           same           same

White Down Arrow           50/00           same           same

White PgDn                 51/00           same           same

White Ins                  52/00           same           same

White Del                  53/00           same           same

SysReq                     no key           **             **

Key 45 [5]                 no key         56/5C           same

Enter (number keypad)      no key         1C/0D          E0/0D

Gray /                     no key         35/2F          E0/2F

PrtSc                      no key           **             **

Pause                      no key           **             **

Gray Home                  no key         47/00          47/E0

Gray Up Arrow              no key         48/00          48/E0

Gray Page Up               no key         49/00          49/E0

Gray Left Arrow            no key         4B/00          4B/E0

Gray Right Arrow           no key         4D/00          4D/E0

Gray End                   no key         4F/00          4F/E0

Gray Down Arrow            no key         50/00          50/E0

Gray Page Down             no key         51/00          51/E0

Gray Insert                no key         52/00          52/E0

Gray Delete                no key         53/00          53/E0

Shift Esc                  01/1B           same           same

!                          02/21           same           same

@                          03/40           same           same

#                          04/23           same           same

$                          05/24           same           same

%                          06/25           same           same

^                          07/5E          same           same

&                          08/26           same           same

* (white)                  09/2A           same           same

(                          0A/28           same           same

)                          0B/29           same           same

_                          0C/5F           same           same

+ (white)                  0D/2B           same           same

Shift Backspace            0E/08           same           same

Shift Tab (Backtab)        0F/00           same           same

Q                          10/51           same           same

W                          11/57           same           same

E                          12/45           same           same

R                          13/52           same           same

T                          14/54           same           same

Y                          15/59           same           same

U                          16/55           same           same

I                          17/49           same           same

O                          18/4F           same           same

P                          19/50           same           same

{                          1A/7B           same           same

}                          1B/7D           same           same

Shift Enter                1C/0D           same           same

Shift Ctrl                   **             **             **

A                          1E/41           same           same

S                          1F/53           same           same

D                          20/44           same           same

F                          21/46           same           same

G                          22/47           same           same

H                          23/48           same           same

J                          24/4A           same           same

K                          25/4B           same           same

L                          26/4C           same           same

:                          27/3A           same           same

"                          28/22           same           same

~                          29/7E           same           same

|                          2B/7C           same           same

Z                          2C/5A           same           same

X                          2D/58           same           same

C                          2E/43           same           same

V                          2F/56           same           same

B                          30/42           same           same

N                          31/4E           same           same

M                          32/4D           same           same

<                          33/3C           same           same

>                          34/3E           same           same

?                          35/3F           same           same

Shift Gray *                 **             **           37/2A

Shift Alt                    **             **             **

Shift Space                39/20           same           same

Shift Caps Lock              **             **             **

Shift F1                   54/00           same           same

Shift F2                   55/00           same           same

Shift F3                   56/00           same           same

Shift F4                   57/00           same           same

Shift F5                   58/00           same           same

Shift F6                   59/00           same           same

Shift F7                   5A/00           same           same

Shift F8                   5B/00           same           same

Shift F9                   5C/00           same           same

Shift F10                  5D/00           same           same

Shift F11                  no key           --           87/00

Shift F12                  no key           --           88/00

Shift Num Lock               **             **             **

Shift Scroll Lock            **             **             **

Shift 7 (number pad)       47/37           same           same

Shift 8 (number pad)       48/38           same           same

Shift 9 (number pad)       49/39           same           same

Shift Gray -               4A/2D           same           same

Shift 4 (number pad)       4B/34           same           same

Shift 5 (number pad)       4C/35           same           same

Shift 6 (number pad)       4D/36           same           same

Shift Gray +               4E/2B           same           same

Shift 1 (number pad)       4F/31           same           same

Shift 2 (number pad)       50/32           same           same

Shift 3 (number pad)       51/33           same           same

Shift 0 (number pad)       52/30           same           same

Shift . (number pad)       53/2E           same           same

Shift SysReq               no key           **             **

Shift Key 45 [5]           no key         56/7C           same

Shift Enter (number pad)   no key         1C/0D          E0/0D

Shift Gray /               no key         35/2F          E0/2F

Shift PrtSc                no key           **             **

Shift Pause                no key           **             **

Shift Gray Home            no key         47/00          47/E0

Shift Gray Up Arrow        no key         48/00          48/E0

Shift Gray Page Up         no key         49/00          49/E0

Shift Gray Left Arrow      no key         4B/00          4B/E0

Shift Gray Right Arrow     no key         4D/00          4D/E0

Shift Gray End             no key         4F/00          4F/E0

Shift Gray Down Arrow      no key         50/00          50/E0

Shift Gray Page Down       no key         51/00          51/E0

Shift Gray Insert          no key         52/00          52/E0

Shift Gray Delete          no key         53/00          53/E0

Ctrl Esc                   01/1B           same           same

Ctrl 1                       --             --             --

Ctrl 2 (NUL)               03/00           same           same

Ctrl 3                       --             --             --

Ctrl 4                       --             --             --

Ctrl 5                       --             --             --

Ctrl 6 (RS)                07/1E           same           same

Ctrl 7                       --             --             --

Ctrl 8                       --             --             --

Ctrl 9                       --             --             --

Ctrl 0                       --             --             --

Ctrl -                     0C/1F           same           same

Ctrl =                       --             --             --

Ctrl Backspace (DEL)       0E/7F           same           same

Ctrl Tab                     --             --           94/00

Ctrl q (DC1)               10/11           same           same

Ctrl w (ETB)               11/17           same           same

Ctrl e (ENQ)               12/05           same           same

Ctrl r (DC2)               13/12           same           same

Ctrl t (DC4)               14/14           same           same

Ctrl y (EM)                15/19           same           same

Ctrl u (NAK)               16/15           same           same

Ctrl i (HT)                17/09           same           same

Ctrl o (SI)                18/0F           same           same

Ctrl p (DEL)               19/10           same           same

Ctrl [ (ESC)               1A/1B           same           same

Ctrl ] (GS)                1B/1D           same           same

Ctrl Enter (LF)            1C/0A           same           same

Ctrl a (SOH)               1E/01           same           same

Ctrl s (DC3)               1F/13           same           same

Ctrl d (EOT)               20/04           same           same

Ctrl f (ACK)               21/06           same           same

Ctrl g (BEL)               22/07           same           same

Ctrl h (Backspace)         23/08           same           same

Ctrl j (LF)                24/0A           same           same

Ctrl k (VT)                25/0B           same           same

Ctrl l (FF)                26/0C           same           same

Ctrl ;                       --             --             --

Ctrl '                       --             --             --

Ctrl `                       --             --             --

Ctrl Shift                   **             **             **

Ctrl \ (FS)                2B/1C           same           same

Ctrl z (SUB)               2C/1A           same           same

Ctrl x (CAN)               2D/18           same           same

Ctrl c (ETX)               2E/03           same           same

Ctrl v (SYN)               2F/16           same           same

Ctrl b (STX)               30/02           same           same

Ctrl n (SO)                31/0E           same           same

Ctrl m (CR)                32/0D           same           same

Ctrl ,                       --             --             --

Ctrl .                       --             --             --

Ctrl /                       --             --             --

Ctrl Gray *                  --             --           96/00

Ctrl Alt                     **             **             **

Ctrl Space                 39/20           same           same

Ctrl Caps Lock               --             --             --

Ctrl F1                    5E/00           same           same

Ctrl F2                    5F/00           same           same

Ctrl F3                    60/00           same           same

Ctrl F4                    61/00           same           same

Ctrl F5                    62/00           same           same

Ctrl F6                    63/00           same           same

Ctrl F7                    64/00           same           same

Ctrl F8                    65/00           same           same

Ctrl F9                    66/00           same           same

Ctrl F10                   67/00           same           same

Ctrl F11                   no key           --           89/00

Ctrl F12                   no key           --           8A/00

Ctrl Num Lock                --             --             --

Ctrl Scroll Lock             --             --             --

Ctrl White Home            77/00           same           same

Ctrl White Up Arrow          --             --           8D/00

Ctrl White PgUp            84/00           same           same

Ctrl Gray -                  --             --           8E/00

Ctrl White Left Arrow      73/00           same           same

Ctrl 5 (number pad)          --             --           8F/00

Ctrl White Right Arrow     74/00           same           same

Ctrl Gray +                  --             --           90/00

Ctrl White End             75/00          75/00           same

Ctrl White Down Arrow        --             --           91/00

Ctrl White PgDn            76/00           same           same

Ctrl White Ins               --             --           92/00

Ctrl White Del               --             --           93/00

Ctrl SysReq                no key           **             **

Ctrl Key 45 [5]            no key           --             --

Ctrl Enter (number pad)    no key         1C/0A          E0/0A

Ctrl / (number pad)        no key           --           95/00

Ctrl PrtSc                 no key         72/00           same

Ctrl Break                 no key         00/00           same

Ctrl Gray Home             no key         77/00          77/E0

Ctrl Gray Up Arrow         no key           --           8D/E0

Ctrl Gray Page Up          no key         84/00          84/E0

Ctrl Gray Left Arrow       no key         73/00          73/E0

Ctrl Gray Right Arrow      no key         74/00          74/E0

Ctrl Gray End              no key         75/00          75/E0

Ctrl Gray Down Arrow       no key           --           91/E0

Ctrl Gray Page Down        no key         76/00          76/E0

Ctrl Gray Insert           no key           --           92/E0

Ctrl Gray Delete           no key           --           93/E0

Alt Esc                      --             --           01/00

Alt 1                      78/00           same           same

Alt 2                      79/00           same           same

Alt 3                      7A/00           same           same

Alt 4                      7B/00           same           same

Alt 5                      7C/00           same           same

Alt 6                      7D/00           same           same

Alt 7                      7E/00           same           same

Alt 8                      7F/00           same           same

Alt 9                      80/00           same           same

Alt 0                      81/00           same           same

Alt -                      82/00           same           same

Alt =                      83/00           same           same

Alt Backspace                --             --           0E/00

Alt Tab                      --             --           A5/00

Alt q                      10/00           same           same

Alt w                      11/00           same           same

Alt e                      12/00           same           same

Alt r                      13/00           same           same

Alt t                      14/00           same           same

Alt y                      15/00           same           same

Alt u                      16/00           same           same

Alt i                      17/00           same           same

Alt o                      18/00           same           same

Alt p                      19/00           same           same

Alt [                        --             --           1A/00

Alt ]                        --             --           1B/00

Alt Enter                    --             --           1C/00

Alt Ctrl                     **             **             **

Alt a                      1E/00           same           same

Alt s                      1F/00           same           same

Alt d                      20/00           same           same

Alt f                      21/00           same           same

Alt g                      22/00           same           same

Alt h                      23/00           same           same

Alt j                      24/00           same           same

Alt k                      25/00           same           same

Alt l                      26/00           same           same

Alt ;                        --             --           27/00

Alt '                        --             --           28/00

Alt `                        --             --           29/00

Alt Shift                    **             **             **

Alt \                        --             --           2B/00

Alt z                      2C/00           same           same

Alt x                      2D/00           same           same

Alt c                      2E/00           same           same

Alt v                      2F/00           same           same

Alt b                      30/00           same           same

Alt n                      31/00           same           same

Alt m                      32/00           same           same

Alt ,                        --             --           33/00

Alt .                        --             --           34/00

Alt /                        --             --           35/00

Alt Gray *                   --             --           37/00

Alt Space                  39/20           same           same

Alt Caps Lock                **             **             **

Alt F1                     68/00           same           same

Alt F2                     69/00           same           same

Alt F3                     6A/00           same           same

Alt F4                     6B/00           same           same

Alt F5                     6C/00           same           same

Alt F6                     6D/00           same           same

Alt F7                     6E/00           same           same

Alt F8                     6F/00           same           same

Alt F9                     70/00           same           same

Alt F10                    71/00           same           same

Alt F11                    no key           --           8B/00

Alt F12                    no key           --           8C/00

Alt Num Lock                 **             **             **

Alt Scroll Lock              **             **             **

Alt Gray -                   --             --           4A/00

Alt Gray +                   --             --           4E/00

Alt 7 (number pad)           #              #              #

Alt 8 (number pad)           #              #              #

Alt 9 (number pad)           #              #              #

Alt 4 (number pad)           #              #              #

Alt 5 (number pad)           #              #              #

Alt 6 (number pad)           #              #              #

Alt 1 (number pad)           #              #              #

Alt 2 (number pad)           #              #              #

Alt 3 (number pad)           #              #              #

Alt Del                      --             --             --

Alt SysReq                 no key           **             **

Alt Key 45 [5]             no key           --             --

Alt Enter (number pad)     no key           --           A6/00

Alt / (number pad)           --             --           A4/00

Alt PrtSc                  no key           **             **

Alt Pause                  no key           **             **

Alt Gray Home              no key           --           97/00

Alt Gray Up Arrow          no key           --           98/00

Alt Gray Page Up           no key           --           99/00

Alt Gray Left Arrow        no key           --           9B/00

Alt Gray Right Arrow       no key           --           9D/00

Alt Gray End               no key           --           9F/00

Alt Gray Down Arrow        no key           --           A0/00

Alt Gray Page Down         no key           --           A1/00

Alt Gray Insert            no key           --           A2/00

Alt Gray Delete            no key           --           A3/00

--------------------------------------------------------------------------

Footnotes

[1]   The 83-key and 84-key keyboards are the original PC keyboard

and the original-layout AT keyboard, respectively.

[2]   The 101/102-key keyboard is the current IBM standard

("Enhanced") keyboard.

[3]   "Standard Function" refers to Interrupt 16h Services 0, 1,

and 2.

[4]   "Extended Function" refers to Interrupt 16h Services 10h,

11h, and 12h.

[5]   In the United States, the 101/102-key keyboard is shipped

with 101 keys. Overseas versions have an additional key

sandwiched between the left Shift key and the Z key. This

additional key is identified by IBM (and in this table) as

"Key 45."

[**]  Keys and key combinations marked ** are used by the ROM BIOS

but do not put values into the keyboard buffer.

[--]  Keys and key combinations marked -- are ignored by the ROM

BIOS.

INT 16h,  10h (16)       Extended Keyboard Read                          many

Returns the next character in the keyboard buffer; if no character is

available, this service waits until one is available. This service is

provided to support the enhanced (101/102-key) keyboard.

On entry:      AH         10h

Returns:       AL         ASCII character code

AH         Scan code

--------------------------------------------------------------------------

Notes:         This service is supported only on the AT dated

11/15/85 and after, PC XT dated 1/10/86 and after,

and XT-286.

The scan codes are the numbers representing the

location of the key on the keyboard. As new keys

have been added and the keyboard layout rearranged,

this numbering scheme has not been consistent with

its original purpose. See the list of scan codes.

If the character is a special character, AL will be

0 and the value in AH will be the extended scan code

for the key. See the list of scan codes.

Use the scan codes to differentiate between keys

representing the same ASCII code, such as the plus

key across the top of the keyboard and the gray plus

key.

After the character has been removed from the

keyboard buffer, the keyboard buffer start pointer

(at 0:041Ah) is increased by 2. If the start pointer

is beyond the end of the buffer, the start pointer

is reset to the start of the keyboard buffer.

If no character is available at the keyboard, an INT

15h, Service 90h (Device Busy), will be issued for

the keyboard, informing the operating system that

there is a keyboard loop taking place and thereby

allowing the operating system to perform another

task.

After every character is typed, an INT 15h, Service

91h (Interrupt Complete), is issued. This allows the

operating system to switch back to a task that is

waiting for a character at the keyboard.

See Service 00h for an equivalent service that works

with all keyboards, not supporting the new keys on

the enhanced (101/102-key) keyboard.

INT 16h,  05h (5)        Keyboard Write                                  many

Puts a scan code/character code combination in the keyboard buffer.

On entry:      AH         05h

CH         Scan code

CL         ASCII character code

Returns:       AL         00h - Operation successful

01h - Buffer full

--------------------------------------------------------------------------

Notes:         This service is available only for ATs dated

11/15/85 and after, and XT 286s.

This service "fools" a program into thinking that

keys have come from the keyboard. A good example of

this would be the operation needed by a program that

implements keyboard macros.

For a description and list of scan codes, see "Scan

Codes."

INT 16h,  12h (18)       Get Extended Shift Status                       many

Returns the current keyboard shift status. This service is provided to

support the enhanced (101/102-key) keyboard.

On entry:      AH         12h

Returns:       AL         Shift status

AH         Extended shift status

--------------------------------------------------------------------------

Shift Status (AL)

7 6 5 4 3 2 1 0

1 . . . . . . .      Insert locked

. 1 . . . . . .      Caps Lock locked

. . 1 . . . . .      Num Lock locked

. . . 1 . . . .      Scroll Lock locked

. . . . 1 . . .      Alt key is pressed

. . . . . 1 . .      Ctrl key is pressed

. . . . . . 1 .      Left Shift key is pressed

. . . . . . . 1      Right Shift key is pressed

Extended Shift Status (AH)

7 6 5 4 3 2 1 0

1 . . . . . . .      Sys Req key is pressed

. 1 . . . . . .      Caps Lock key is pressed

. . 1 . . . . .      Num Lock key is pressed

. . . 1 . . . .      Scroll Lock key is pressed

. . . . 1 . . .      Right Alt key is pressed

. . . . . 1 . .      Right Ctrl key is is pressed

. . . . . . 1 .      Left Alt key is pressed

. . . . . . . 1      Left Ctrl key is pressed

Notes:         This service is supported only on the AT dated

11/15/85 and after, the XT dated 1/10/86 and after,

and the XT-286.

The shift status byte return in AL is obtained from

the byte at memory location 0:0417h. Note that this

is the same value that is returned by Service 02h.

Note that the Extended Shift Status (value returned

in AH) reports if keys are being pressed, not

locked.

See Service 02h for an equivalent service that works

with all keyboards, not supporting the new keys on

the enhanced (101/102-key) keyboard.

INT 1Ah,  00h (0)        Read System-Timer Time Counter                   all

Reports the current time of day, and whether 24 hours has passed since

1) the last power-on, 2) the last system reset, or 3) the last system-

timer time read or set.

On entry:      AH         00h

Returns:       CX         High-order part of clock count

DX         Low-order part of clock count

AL         0 if 24 hours has not passed; else 1

--------------------------------------------------------------------------

Notes:         The following formulas convert the clock count to

the time of day:

Hour      = Clock / 65543 (1007h)

Remainder = Clock MOD 65543

Minutes   = Remainder / 1092 (444h)

Remainder = Remainder MOD 1092

Second    = Remainder / 18.21

Remainder = Remainder MOD 18.21

Hundredths = CINT(Remainder * 100)

The "system timer" (as distinguished from the real-

time clock) is the timer that's set when the system

is started. This time is temporary, lasting only as

long as the system is turned on.

The clock count may also be read as a 4-byte integer

at memory location 0:046C. This 4-byte value is

equal to the 4-byte integer in CX:DX after Service

00h has been called.

After the call, the flag (at 0:0470h) stating

whether 24 hours has passed or not, is cleared.

When TIME is typed at the command line, DOS gets the

time by means of this service.

Counts occur at the rate of 18.2 per second.

INT 1Ah,  01h (1)        Set System-Timer Time Counter                    all

Sets the current time of day.

On entry:      AH         01h

CX         High-order part of clock count

DX         Low-order part of clock count

Returns:       None

--------------------------------------------------------------------------

Notes:         The following formula converts the time of day to a

clock count:

Count = (Hout * 65543.33) + (Minutes * 1092.38) +

(Seconds * 18.21) + (Hundreths * .182)

The "system timer" (as distinguished from the real-

time clock) is the timer that's set when the system

is started. This time is temporary, lasting only as

long as the system is turned on.

The clock count may also be set as a 4-byte integer

at memory location 0:046C. This 4-byte value will be

set to the 4-byte integer in CX:DX after the call.

After the set, the flag (at 0:0470h) stating whether

24 hours has passed or not, is cleared.

When TIME is typed at the command line, DOS gets the

time by means of this service. Setting a new time

will call this service.

Counts occur at the rate of 18.2 per second.

INT 1Ah,  02h (2)        Read Real-Time Clock Time                       many

Reads the time from the computer's real-time clock.

On entry:      AH         02h

Returns:       CF         Set if clock not operating; else cleared

CH         Hours (BCD)

CL         Minutes (BCD)

DH         Seconds (BCD)

DL         1 if daylight saving time option; else 0

--------------------------------------------------------------------------

Notes:         All times are in Binary Coded Decimal (BCD). For

example, 9:04.12 am will be reported as CX = 0904;

DH = 12.

The real-time clock is the clock that runs even when

the computer is turned off. A CMOS battery is used

so that, even while the computer is off, the date,

time, and alarm time are maintained.

This service is available only for ATs, XT-286s, and

PC Convertibles. Previous machines will report

unpredictable results (including the state of the

Carry Flag).

The daylight saving time option is not reported on

ATs with dates prior to 6/10/85.

The information returned from this service could

differ from the system time. This is because the

real-time clock is read once upon system startup,

updating the system timer. All other references to

time use the system timer. If either the real-time

clock is changed (via Service 03h) or the system

timer is changed (via Service 01h), the other clock

is not changed accordingly.

Even though the system uses the system timer for

timing information, both the real-time clock and the

system timer are updated continuously.

INT 1Ah,  03h (3)        Set Real-Time Clock Time                        many

Sets the time for the computer's real-time clock.

On entry:      AH         03h

CH         Hours (BCD)

CL         Minutes (BCD)

DH         Seconds (BCD)

DL         1 if daylight saving time option; else 0

Returns:       None

--------------------------------------------------------------------------

Notes:         All times are in Binary Coded Decimal (BCD). For

example, 9:04.12 am would be set as CX = 0904; DH =

12.

The real-time clock is the clock that runs even when

the computer is turned off. A CMOS battery is used

so that, even while the computer is off, the date,

time, and alarm time are maintained.

This service is available only for ATs, XT-286s, and

PC Convertibles. Previous machines will report

unpredictable results (including the state of the

Carry Flag).

Setting the real-time clock will not affect the

system time. This is because the real-time clock is

read once upon system startup, updating the system

timer. All other references to time use the system

timer. If either the real-time clock is changed (via

this service) or the system timer is changed (via

Service 01h), the other clock is not changed

accordingly.

Even though the system uses the system-timer for

timing information, both the real-time clock and the

system timer are updated continuously.

INT 1Ah,  04h (4)        Read Real-Time Clock Date                       many

Reads the date from the computer's real-time clock.

On entry:      AH         04h

Returns:       CF         Set if clock not operating; else cleared

CH         Century (19 or 20) (BCD)

CL         Year (BCD)

DH         Month (BCD)

DL         Day (BCD)

--------------------------------------------------------------------------

Notes:         All dates are in Binary Coded Decimal (BCD). For

example, November 15, 1987 will be reported as CX =

1987; DX = 1115.

The real-time clock is the clock that runs even when

the computer is turned off. A CMOS battery is used

so that, even while the computer is off, the date,

time, and alarm time are maintained.

This service is available only for ATs, XT-286s, and

PC Convertibles. Previous machines will report

unpredictable results (including the state of the

Carry Flag).

The information returned from this service could

differ from the system date. This is because the

real-time clock is read once upon system startup,

updating the system timer. All other references to

date use the system timer. If either the real-time

clock date is changed (via Service 05h) or the

system timer is changed (via Service 01h), the other

date is not changed accordingly.

Even though the system uses the system timer for

date information, both the real-time clock and the

system timer are updated continuously.

INT 1Ah,  05h (5)        Set Real-Time Clock Date                        many

Sets the date on the computer's real-time clock.

On entry:      AH         05h

CH         Century (19 or 20) (BCD)

CL         Year (BCD)

DH         Month (BCD)

DL         Day (BCD)

--------------------------------------------------------------------------

Notes:         All dates are in Binary Coded Decimal (BCD). For

example, January 15, 1987 will be set as CX = 1987;

DX = 0115.

The real-time clock is the clock that runs even when

the computer is turned off. A CMOS battery is used

so that, even while the computer is off, the date,

time, and alarm time are maintained.

This service is available only for ATs, XT-286s, and

PC Convertibles. Previous machines will report

unpredictable results (including the state of the

Carry Flag).

Setting the real-time clock date will not affect the

system date. This is because the real-time clock is

read once upon system startup, updating the system

timer. All other references to date use the system

timer. If either the real-time clock date is changed

(via this service) or the system timer is changed

(via Service 01h), the other date is not changed

accordingly.

Even though the system uses the system timer for

date information, both the real-time clock and the

system timer are updated continuously.

INT 08h (8)              System Timer

This interrupt is a hardware interrupt (IRQ 0) activated by the system

timer 18.2 times per second (every 55 ms). The default handler does

the following:

ş   Keeps a count of the number of timer ticks at memory location

0:46Ch (System Timer Counter--long integer). After 24 hours of

operation, a flag is set at memory location 0:470h to signal

this condition and the System Timer Counter (0:46Ch) is reset to

0.

ş   Decrements the Diskette Drive Motor Off Counter at memory

location 0:440h if it is not 0. When this location reaches 0,

the diskette drive motor is turned off and the Diskette Drive

Motor Status byte at memory location 0:43Fh is updated to

reflect that the motor has been turned off.

ş   Generates INT 1Ch.

ş   For the PC Convertible, generates INT 4Ah if an alarm interrupt

occurs.

INT 1Ch (28)             User Timer Tick

Called at every timer tick (18.2 times per second, or every 55 ms) by

INT 08h (System Timer).

--------------------------------------------------------------------------

By default, the interrupt handler for this interrupt IRETs back to the

caller. The user can supply his own handler so he can attain control

at every timer tick.

Notes:         Note that this interrupt is called by the hardware

timer interrupt (INT 08h), which has not signaled

the end of the interrupt back to the interrupt

controller. Therefore, this routine is limited in

the actions it can take, since all interrupts will

be disabled. A better approach to this problem is to

intercept INT 08h. The new INT 08h handler will

first call the standard INT 08h, which will handle

the interrupt controller completion signals

mentioned above. Then after the standard INT 08h is

completed, the new handler can do whatever it has to

do at every timer tick.

INT 70h (112)            Real-Time Clock              XT-286, AT, Convertible

Handles the alarm and periodic interrupts for the real-time clock.

--------------------------------------------------------------------------

This is a hardware interrupt (IRQ 8) activated by the CMOS timer 1024

times per second.

Notes:         This interrupt is available only on the AT and XT-

286. The real-time clock on the PC Convertible

generates an INT 02h (Non-Maskable Interrupt) to

handle the periodic and alarm functions.

This interrupt provides two services. The first is

the periodic interrupt that is generated when a time

delay is specified. See INT 15h, Service 83h (Event

Wait), and Service 86h (Wait), for more information.

The second service is the Alarm Interrupt. When the

CMOS alarm time matches the current CMOS time, then

INT 4Ah (User Alarm) is executed.

INT 1Bh (27)             Keyboard Break

Called when a Control-Break key sequence is encountered.

--------------------------------------------------------------------------

When the machine is powered on, the interrupt handler for this

interrupt just IRETs back to the caller. DOS, however, installs a

Ctrl-Break handler that sets an internal flag to signal that a

keyboard Break condition has occurred.

Notes:         DOS tests the state of the internal flag

occasionally, depending on the state of the Control-

Break Check (INT 21h, Function 33h). If the flag is

on during a test, then the string "^C", followed by

a Carriage-Return/ Line-Feed combination, is

displayed on the screen, the DOS internal flag is

reset, and INT 23h (Control-Break Exit Address) is

called.

Note that INT 09h (Keyboard) sets the byte at

0:0471h to signal that a Control-Break key has been

entered.

INT 15h,  85h (133)      System Request Pressed                          many

Provides an interface for special processing of the Sys Req key.

On entry:      AH         85h

AL         Subservice

00h - System Request key Make (pressed)

01h - System Request key Break (released)

Returns:       Carry      Set if service not supported, else cleared

--------------------------------------------------------------------------

By default, this service does nothing but clear the Carry Flag. It is

up to the operating system or user to provide a handler to process the

Sys Req key. This service is called by INT 09h (Keyboard) after the

Sys Req key has been pressed ("made") or released ("broken").

Notes:         The PC, PCjr, and the XT dated 11/08/82 do not

support this service.

Currently there is no standard use for the System

Request key. Future operating systems will probably

use it to switch between tasks.

INT 15h,  83h (131)      Event Wait                                      many

Modifies a byte in the user's memory after a specified time interval.

Control is transferred to the caller immediately after this service is

called.

On entry:      AH         83h

AL         Subservice

00h - Set interval

01h - Cancel set interval

ES:BX - Pointer to byte that will be posted

CX:DX - Microseconds until posting (long integer)

Returns:       Carry      Set if service not supported, else cleared

--------------------------------------------------------------------------

After this service is called, control is returned immediately to the

caller. This service does not work by delaying and then returning

control to the user, but returns control immediately; the user must

periodically look at the byte specified (pointed to by ES:BX) to

determine when the interval has elapsed.

Notes:         The PC, PCjr, and the XT dated 11/08/82 do not

support this service. The AT dated 1/10/84 is only

able to set the interval; thus the value in AL is

not used.

The byte is posted by setting the high bit (bit 7).

To test the end of the interval, make sure the high

bit of the byte is clear before calling Subservice 0

(Set Interval).

The real-time clock is used to count the number of

microseconds.

The granularity of the time is 976 microseconds.

CX is the high-order word of the count. For example,

if CX is 98h and DX is 9680h, then a 10-second delay

would be specified.

INT 15h,  91h (139)      Interrupt Complete                              many

Signals the system that a device is ready to be serviced.

On entry:      AH         91h

AL         Device type (See service 90h)

Returns:       Carry      Set if error, else cleared

--------------------------------------------------------------------------

Notes:         This service is not available for the PC, PCjr, and

XT dated 11/08/82.

When a device is needed by a process, but access to

it is delayed, the system can perform another task

to achieve maximum performance until the device is

freed up for the process to use. In a multi-tasking

environment, that is the purpose for this service.

The operating system can keep track of which process

is waiting for which device, and perform another

task while the device is busy. After a device is

ready to be serviced, then this function is called

to notify the system that a device is ready.

For example, INT 09h will call this service after a

key has been entered, to signal the operating system

that a key has been entered. Thus if the system was

performing another task while a previous process was

waiting for a keyboard character, then the previous

process could regain control of the system and get

the keyboard character just entered.

INT 14h,  00h (00)       Initialize Serial Port Parameters

Initializes the baud rate, parity, stop-bit, and word length

parameters for a serial port, and returns the status for the port.

On entry:      AH         00h

AL         Communications parameters (see below)

DX         Serial port number (0 - COM1, 1 - COM2, etc.)

Returns:       AX         Line and modem status (see Service 03h)

--------------------------------------------------------------------------

Baud rate           Parity           Stop bits      Word Length

Bit:    7 6 5               4 3              2              1 0

0 0 0    110        0 0  None        0  One         1 0  7 bits

0 0 1    150        0 1  Odd         1  Two         1 1  8 bits

0 1 0    300        1 0  None

0 1 1    600        1 1  Even

1 0 0  1,200

1 0 1  2,400

1 1 0  4,800

1 1 1  9,600

INT 14h,  03h (3)        Get Serial Port Status

Returns line status and modem status information for a specified

serial port.

On entry:      AH         03h

DX         Serial port number (0 - COM1, 1 - COM2, etc.)

Returns:       AX         Line and Modem status (See below)

--------------------------------------------------------------------------

Status information is returned in AX, as follows:

AH (Line status)

7 6 5 4 3 2 1 0

1 . . . . . . .       Time-out error

. 1 . . . . . .       Transfer shift register empty

. . 1 . . . . .       Transfer holding register empty

. . . 1 . . . .       Break-detect error

. . . . 1 . . .       Framing error

. . . . . 1 . .       Parity error

. . . . . . 1 .       Overrun error

. . . . . . . 1       Data ready

AL (Modem status)

7 6 5 4 3 2 1 0

1 . . . . . . .       Received line signal detect

. 1 . . . . . .       Ring indicator

. . 1 . . . . .       Data set ready

. . . 1 . . . .       Clear to send

. . . . 1 . . .       Change in receive line signal detected

. . . . . 1 . .       Trailing edge ring detector

. . . . . . 1 .       Change in data set ready

. . . . . . . 1       Change in clear to send

If bit 7 of the Line status byte (AH) is set, then the rest of the

bits in AH and AL are unpredictable.

Notes:         Early versions of the ROM-BIOS for the original PC

had a programming error that would cause "time-out"

errors to be reported as "transfer shift register

empty" and "break-detect" errors. This has been

corrected in all other versions of the ROM-BIOS.

INT 14h,  01h (1)        Send One Character

Sends one character to the specified serial port.

On entry:      AH         01h

AL         Character

DX         Serial port number (0 - COM1, 1 - COM2, etc.)

Returns:       AH         Line status (see Service 03h)

--------------------------------------------------------------------------

If an error occurs, bit 7 of AH will be set. SInce bit 7 is used as a

general error flag, this service is unable to identify a time-out

error. For complete diagnostic information, use Service 03h (Get

serial port status).

INT 14h,  02h (2)        Receive One Character

Receives one character at the specified serial port.

On entry:      AH         02h

DX         Serial port number (0 - COM1, 1 - COM2, etc.)

Returns:       AL         Character

AH         Line status (See service 03h)

--------------------------------------------------------------------------

Notes:         This service waits for a character. If no character

is available or an error occurs, no character is

returned and bit 7 of AH is set.

If an error occurs, bit 7 of AH is set. SInce bit 7

is used as a general error flag, this service is

unable to identify a time-out error. For complete

diagnostic information, use Service 03h (Get serial

port status).