Code:

;       Reading the harddisk using ports!
;       +-------------------------------+   by qark
;
;
;  This took me months to get working but I finally managed it.
;
;  This code only works for the 286+ so you must detect for 8088's somewhere
;  in your code.
;
;  Technical Information on the ports:
;      Port    Read/Write   Misc
;     ------  ------------ -------------------------------------------------
;       1f0       r/w       data register, the bytes are written/read here
;       1f1       r         error register  (look these values up yourself)
;       1f2       r/w       sector count, how many sectors to read/write
;       1f3       r/w       sector number, the actual sector wanted
;       1f4       r/w       cylinder low, cylinders is 0-1024
;       1f5       r/w       cylinder high, this makes up the rest of the 1024
;       1f6       r/w       drive/head
;                              bit 7 = 1
;                              bit 6 = 0
;                              bit 5 = 1
;                              bit 4 = 0  drive 0 select
;                                    = 1  drive 1 select
;                              bit 3-0    head select bits
;       1f7       r         status register
;                              bit 7 = 1  controller is executing a command
;                              bit 6 = 1  drive is ready
;                              bit 5 = 1  write fault
;                              bit 4 = 1  seek complete
;                              bit 3 = 1  sector buffer requires servicing
;                              bit 2 = 1  disk data read corrected
;                              bit 1 = 1  index - set to 1 each revolution
;                              bit 0 = 1  previous command ended in an error
;       1f7       w         command register
;                            commands:
;                              50h format track
;                              20h read sectors with retry
;                              21h read sectors without retry
;                              22h read long with retry
;                              23h read long without retry
;                              30h write sectors with retry
;                              31h write sectors without retry
;                              32h write long with retry
;                              33h write long without retry
;

;  Most of these should work on even non-IDE hard disks.
;  This code is for reading, the code for writing is the next article.

   mov     dx,1f6h         ;Drive and head port
   mov     al,0a0h         ;Drive 0, head 0
   out     dx,al

   mov     dx,1f2h         ;Sector count port
   mov     al,1            ;Read one sector
   out     dx,al

   mov     dx,1f3h         ;Sector number port
   mov     al,1            ;Read sector one
   out     dx,al

   mov     dx,1f4h         ;Cylinder low port
   mov     al,0            ;Cylinder 0
   out     dx,al

   mov     dx,1f5h         ;Cylinder high port
   mov     al,0            ;The rest of the cylinder 0
   out     dx,al

   mov     dx,1f7h         ;Command port
   mov     al,20h          ;Read with retry.
   out     dx,al
still_going:
   in      al,dx
   test    al,8            ;This means the sector buffer requires
            ;servicing.
   jz      still_going     ;Don't continue until the sector buffer
            ;is ready.

   mov     cx,512/2        ;One sector /2
   mov     di,offset buffer
   mov     dx,1f0h         ;Data port - data comes in and out of here.
   rep     insw

;   ------
   mov     ax,201h         ;Read using int13h then compare buffers.
   mov     dx,80h
   mov     cx,1
   mov     bx,offset buffer2
   int     13h

   mov     cx,512
   mov     si,offset buffer
   mov     di,offset buffer2
   repe    cmpsb
   jne     failure

mov     ah,9
   mov     dx,offset readmsg
   int     21h
   jmp     good_exit
failure:
   mov     ah,9
   mov     dx,offset failmsg
   int     21h
good_exit:
   mov     ax,4c00h        ;Exit the program
   int     21h

   readmsg db      'The buffers match.  Hard disk read using ports.$'
   failmsg db      'The buffers do not match.$'

buffer  db      512 dup ('V')
buffer2 db      512 dup ('L')

;
;       Writing to the hard disk using the ports!     by qark
;       +---------------------------------------+
;
;  The only differences between reading and writing using the ports is
;  that 30h is sent to the command register, and instead of INSW you
;  OUTSW.  
;

   mov     dx,1f6h         ;Drive and head port
   mov     al,0a0h         ;Drive 0, head 0
   out     dx,al

   mov     dx,1f2h         ;Sector count port
   mov     al,1            ;Write one sector
   out     dx,al

   mov     dx,1f3h         ;Sector number port
   mov     al,1           ;Wrote to sector one
   out     dx,al

   mov     dx,1f4h         ;Cylinder low port
   mov     al,0            ;Cylinder 0
   out     dx,al

   mov     dx,1f5h         ;Cylinder high port
   mov     al,0            ;The rest of the cylinder 0
   out     dx,al

   mov     dx,1f7h         ;Command port
   mov     al,30h          ;Write with retry.
   out     dx,al

oogle:
   in      al,dx
   test    al,8            ;Wait for sector buffer ready.
   jz      oogle

   mov     cx,512/2        ;One sector /2
   mov     si,offset buffer
   mov     dx,1f0h         ;Data port - data comes in and out of here.
   rep     outsw           ;Send it.

;    ------------

   mov     ax,201h                 ;We'll read in sector 1 using
   mov     bx,offset buffer2       ;int13h and see if we are successful.
   mov     cx,1
   mov     dx,80h
   int     13h

   mov     cx,512
   mov     si,offset buffer
   mov     di,offset buffer2
   repe    cmpsb                   ;Compare the buffers.
   jne     failure

   mov     ah,9
   mov     dx,offset write_msg
   int     21h
   jmp     w_exit
failure:
   mov     ah,9
   mov     dx,offset fail
   int     21h

w_exit:
   mov     ax,4c00h        ;Exit the program
   int     21h

   write_msg       db      'Sector one written to using the ports, OH NO! there goes XP.$'
   fail            db      'Writing using ports failed.$'

buffer  db      512 dup ('A')
buffer2 db      512 dup ('D')